This dataset supports the hackathon challenge of estimating a wind turbine’ energy yield loss during periods of missing measurements using data from reference turbines in other wind parks. Teams are free to select individual reference turbines, multiple turbines, or data aggregated at wind-park level, as long as the reference data comes from outside the target wind park. The choice of reference turbines and parks is entirely up to the teams.

- Dataset Structure

The dataset contains

- 90 days of time series data from 10 wind turbines across three wind parks, recorded at 10-minute intervals, as well as

- power curve data of the target turbines.

For differentiating between different turbines, each turbine is named using a capital letter followed by a number (e.g. T1, O2, H3):

- Letter: Identifies the wind park. All turbines in the same park share the same letter.

- Number: Identifies the individual turbine within the park.

- Target turbines: Turbines whose names start with T.

- Reference turbines: All turbines outside the target wind park.

  - Time series data

The dataset for each turbine is provided in a separate file (e.g. T1.xslx) and contains the following parameters:

- Name of the turbine (turbine_name): The turbine identifier consisting of a capital letter followed by a number (not changing within a file)

- Timestamp (timestamp): Date and time of the measurement.

- Wind speed (wind_speed): Represents the average wind conditions of the turbine at hub height.

- Wind direction (wind_direction): May help identify directional effects and differences in exposure between wind parks.

- Ambient temperature (ambient_temperature): Provides information about the surrounding atmospheric conditions.

- Air pressure (air_pressure): May provide additional information about atmospheric conditions and air density.

- Wind speed standard deviation (wind_speed_stddev): Describes the variability of wind speed within the measurement interval and may help characterize turbulent conditions.

- Test period indicator (test_set): Identifies the designated test period within the 90-day dataset

## Important Note:

Wind speed is the primary parameter for the challenge. The use of additional environmental parameters is optional, and teams can decide which variables to incorporate into their solutions.

Please note the following limitations:

- Air pressure and wind speed standard deviation: These parameters are unavailable for one wind park in the dataset (O1, O2). For this park, air pressure has been replaced by a constant value representing the estimated average air pressure at the turbines' hub height. This value does not represent actual time-varying measurements.

- Parameter availability: Solutions using additional environmental parameters must account for the limited availability of those two parameters across wind parks and turbines.

  - Power curve data

The power curves for the turbines in the target wind park are provided separately in files named after the turbine identifier, followed by the suffix _power_curve (e.g., T1_power_curve.xlsx). They describe the relationship between wind speed and expected power output and can be used in combination with the predicted wind speed time series to estimate the expected energy yield loss over a given outage period.

- Training and Test Data

The dataset covers 90 days, with a 10-day test period highlighted in the data. This period is divided into shown and hidden test data.

- Shown test data (4th of September – 9th of September, inclusive): 
Actual measurements are available and should be used for conventional model performance evaluation

- Hidden test data (1st of September – 3rd of September, inclusive): 
Actual measurements are withheld from participants to simulate a real turbine outage. Missing values for the affected turbines in the target wind park are represented by NaN. Teams should estimate these values using available information, including data from their selected reference turbines or wind parks. The judges will compare the submitted predictions against the withheld actual measurements.

- Missing Data and Data Quality

The datasets contain several types of missing data:

- Random missing values: Random gaps have been introduced into the datasets and should be handled appropriately by the proposed solution.

- Communication gaps: Some training datasets contain blocks of consecutive missing measurements of several hours in July caused by actual communication issues. These gaps should be treated as unavailable data, and the solution should be able to handle such periods appropriately without relying on the missing measurements.

- Hidden test measurements: These are deliberately withheld to simulate an outage and must be predicted by the teams.

All missing values are represented by NaN. Missing values should not be interpreted as zero wind speed or zero power output.